B-005

BNC Jack Crimp Bulkhead

| Electrical |  |  |
| --- | --- | --- |
| Impedance | 50Ω | 75Ω |
| Frequency range | 0 to 4GHz |  |
| Voltage rating | 500 volts peak |  |
| Dielectric withstanding voltage | 1500 volts rms |  |
| VSWR | M39012 straight connectors: ors: 1.3 max.0-4GHz M39012 right angle: 1.35 max 0-4GHz |  |
| Other (MIL-C-39012 Cable Connectors) | Contact resistance: Center contact 1.5 milliohm Outer contact 0.2 milliohm Insertion loss: 0.2db max at 3.0 GHz Insulation resistance: 5000 megohms(min) |  |

| Mechanical & Environmental |  |
| --- | --- |
| Temperature range | Teflon insulations: -65 oC to 165 oC Delrin insulations: -55 oC to 85 oC |
| Mating | 2-stud bayonet lock coupling |
| Cable retention | Clamps: 20 to 50 lbs Crimps: 20 to 100 lbs |
| Vibration | MIL-Std-202 method 204(test cond.D) |
| Corrosion | MIL-Std-202 method 101(test cond.B) |

| Material |  |
| --- | --- |
| Connector body | Brass & Zine (Nickel,Silver,Gold,Non nickel plated) |
| Center contact | Plug: Brass Jack: Brass,Phosphor bronze,Beryllium Copper (Silver,Glod,plated) |
| Insulation | Teflon & Delrin |
| Gasker | Rubber,Silicone rubber |
| Crimp ferrule | Copper |