NP-020

Nipple Jack – TNC Jack

| Electrical |  |
| --- | --- |
| Impedance | 50Ω |
| Frequency range | 0 to 3 GHz |
| VSWR (straight connectors) | ≤ 1.22 typ |
| Insulation resistance | ≥ 5¡Ñ103M Ω |
| Center contact resistance | ≤ 10.0M Ω |
| Outer contact resistance | ≤ 5.0M Ω |
| Test voltage | 1000V rms |
| Working voltage | 500V rms |
| Insertion loss(dB) | <0.1dB max/1GHz typ |

| Mechanical & Environmental |  |
| --- | --- |
| Temperature range | Teflon insulations: -65 °C to 165 °C Delrin insulations: -55 °C to 85 °C |
| Mating | M8P0.75 thread coupling |
| Climatical class according IEC | IEC 68 155/55/21 |
| Vibration | MIL-STD-202 method 204/B |
| Corrosion | Salt spray MIL-STD-202 Meth.101 |

| Material |  |
| --- | --- |
| Connector body | Brass (Nickel,Silver,Gold,Non nickel plated) |
| Center contact | Plug: Brass Jack: Brass,Phosphor bronze, (Silver,Glod,plated) |
| Insulation | Teflon & Delrin |
| Crimp ferrule | Copper |