S-023

SMA Plug - N Jack

| Electrical |  |
| --- | --- |
| Frequency range | 0 to 18 GHz |
| Voltage rating | RG-55.58.141.142.223.303: 500 volts peak RG-122.174.188 316.375: 375 volts peak |
| VSWR (straight connectors) | RG-55 group: 1.15+0.011(GHz) RG-178 group: 1.20+0.251(GHz) |
| Contact resistance | Center:2.0 miliohms Body:2.0 miliohms |
|  | Braid to body:0.5 miliohms |
| Insertion resistance | 5000 megohms |
| Insertion loss | 0.03 √f(GHZ) dB max |

| Mechanical & Environmental |  |
| --- | --- |
| Temperature range | Teflon insulations: -65 ˚C to 165 ˚C |
| Mating | 1/4-36 UNS threaded coupling |
| Cable retention | 20 to 40 lbs min |
| Shock | MIL-STD-202 method 213(test cond.1) |
| Vibration | MIL-STD-202 method 204(test cond.D) |
| Corrosion | MIL-STD-202 method 101(test cond.B) |

| Material |  |
| --- | --- |
| Connector body | Brass (Nickel,Silver,Gold,Non nickel plated) |
| Center contact | Plug: Brass Jack: Brass,Phosphor bronze,Beryllium Copper (Glod,plated) |
| Insulation | Teflon |
| Gasker | Rubber,Silicone rubber |
| Crimp ferrule | Copper |