T-033

TNC Jack – Plug – Jack (“T” Type)

| Electrical |  |  |
| --- | --- | --- |
| Impedance | 50 Ω | 75Ω |
| Frequency range | 0 to 11.0 GHz | 0 to 1 GHz |
| Voltage rating | 500 volts peak |  |
| Dielectric withstanding voltage | 1500 volts rms |  |
| VSWR | M39012 straight connectors: 1.3max. 0-11.0 GHz M39012 right angle: 1.35 max. 0-11.0 GHz |  |
| Other (MIL-C-39012 Cable Connectors) | contact resistance: center contact 1.5 milliohm outer contact 0.2 milliohm |  |
|  | Insertion loss: 0.18db max at 9 GHz Insulation resistance: 5000 megohms (min) |  |

| Mechanical & Environmental |  |
| --- | --- |
| Temperature range | Teflon insulations: -65 °C to 165 °C Delrin insulations: -55 °C to 85 °C |
| Mating | 7/16-28 UNEF threaded coupling |
| Cable affixment | Clamps:20 to 50 lbs Crimps: 20to 100 lbs |
| Vibration | MIL-STD-202 method 204 (test cond.D) |
| Corrosion | MIL-STD-202 method 101 (test cond.B) |

| Material |  |
| --- | --- |
| Connector body | Brass (Nickel,Silver,Gold,Non nickel plated) |
| Center contact | Plug: Brass Jack: Brass,Phosphor bronze (Silver,Glod,plated) |
| Insulation | Teflon & Delrin |
| Gasket | Rubber,Silicone rubber |
| Crimp ferrule | Copper |