U-005

UHF Plug Crimp

| Electrical |  |
| --- | --- |
| Impedance | Non-constant |
| Frequency range | 0 to 400 MHz |
| Voltage rating | 500 volts peak |

| Mechanical & Environmental |  |
| --- | --- |
| Temperature range | Teflon insulations: -65 oC to 165 oC Delrin insulations: -55 oC to 85 oC |
| Mating | 5/8-24 UNEF threaded coupling |
| Cable affixment | Braid solder, clamp and crimp |

| Material |  |
| --- | --- |
| Connector body | Brass or Diecast Zinc (Nickel,Silver,Gold plated) |
| Center contact | Plug: Brass (Nickel,Silver,Gold plated) Jack: Brass,Phosphor bronze (Nickel,Silver,Glod,plated) |
| Insulation | Teflon & Delrin |
| Gasket | Rubber,Silicone rubber |
| Crimp ferrule | Copper |